# PiPER Robot Arm: WSL2 and Python Installation Guide

Purpose. This guide records the installation and verification process used to control an AgileX PiPER 6-DOF robot arm from Python on a Windows computer, using Ubuntu under WSL2 and a USB-CAN adapter. The goal is to use pyAgxArm directly, without ROS or ROS2.

Estimated total installation time: approximately 30–90 minutes of hands-on setup, plus potentially longer kernel build/download time depending on computer performance and internet speed. This is an estimate, not a measured duration.

# High-level installation sequence

Prepare Windows and WSL2, and install/launch Ubuntu.

Build a custom WSL Linux kernel with CAN and gs_usb USB-CAN support.

Configure WSL to use the custom kernel and restart WSL.

Attach the USB-CAN adapter to WSL and bring up can0 at 1 Mbps.

Create a Python virtual environment and install python-can and pyAgxArm.

Run a connection test, read arm status and joint angles, and verify repeated feedback.

Important safety note. The verification commands in this document are read-only. Do not run motion commands or uncomment movement examples unless you have reviewed the command, cleared the workspace, secured the arm, removed any load from the gripper, and can reach the emergency stop. Warn the operator before any test that may move the arm, including homing.

# Command-by-command record

Run commands in the order shown. Commands marked “(example)” are templates and may need their paths or values adjusted for the computer being configured. The Windows PowerShell commands that use usbipd should be run in an Administrator PowerShell window where noted.

| Step / command | Terminal | What it does | Expected time |
| --- | --- | --- | --- |
| Open PowerShell as Administrator | Windows PowerShell (Admin) | Allows USB/IP binding and attachment commands. | fast |
| wsl --list --verbose | Windows PowerShell | Checks installed WSL distributions and whether Ubuntu uses WSL 2. | fast |
| wsl --install -d Ubuntu | Windows PowerShell (Admin) | Installs Ubuntu under WSL if it is not already installed; skip if already installed. | Several minutes |
| wsl --shutdown | Windows PowerShell | Stops WSL so kernel configuration changes can take effect. | fast |
| git clone --depth=1 -b linux-msft-wsl-5.15.167.4 https://github.com/microsoft/WSL2-Linux-Kernel.git | Ubuntu (WSL) | Downloads the matching Microsoft WSL kernel source branch. | 1–5 minutes |
| cd ~/WSL2-Linux-Kernel | Ubuntu (WSL) | Moves into the kernel source directory. | fast |
| zcat /proc/config.gz > .config | Ubuntu (WSL) | Uses the currently running kernel configuration as the starting point, if /proc/config.gz is available. | fast |
| make menuconfig | Ubuntu (WSL) | Opens kernel configuration; enable CAN support, CAN_RAW, CAN_DEV, and CAN_GS_USB. Save the configuration. | 5–20 minutes |
| make prepare modules_prepare | Ubuntu (WSL) | Prepares kernel source and headers for building. | 1–5 minutes |
| make -j$(nproc) modules | Ubuntu (WSL) | Builds kernel modules, including the USB-CAN driver. | 5–30 minutes |
| sudo make modules_install | Ubuntu (WSL) | Installs the built modules into the WSL kernel module directory. | 1–5 minutes |
| make -j$(nproc) | Ubuntu (WSL) | Builds the custom Linux kernel image. | 5–30 minutes |
| cp arch/x86/boot/bzImage /mnt/c/Users/<WindowsUser>/wsl_kernel | Ubuntu (WSL) | Copies the built kernel image to a Windows path accessible to WSL configuration. Replace <WindowsUser>. | fast |
| Create %UserProfile%\.wslconfig with kernel=C:\\Users\\<WindowsUser>\\wsl_kernel | Windows (edit file) | Tells WSL to boot the custom kernel. Use the exact Windows path and the [wsl2] section. | fast |
| wsl --shutdown | Windows PowerShell | Restarts WSL cleanly so it loads the configured kernel. | fast |
| uname -r | Ubuntu (WSL) | Verifies that the custom kernel is running; the tested setup showed a version ending in “+”. | fast |
| usbipd list | Windows PowerShell | Finds the USB-CAN adapter and its current BUSID; BUSID can change after reconnecting. | fast |
| usbipd bind --busid <BUSID> | Windows PowerShell (Admin) | Binds the adapter for USB/IP sharing. Usually needed once per device; replace <BUSID>. | fast |
| usbipd attach --wsl --busid <BUSID> | Windows PowerShell (Admin) | Attaches the adapter to WSL. Repeat after detach/reconnect or a WSL restart if necessary. | fast |
| lsusb | Ubuntu (WSL) | Verifies that the USB-CAN adapter is visible inside Ubuntu. | fast |
| sudo modprobe gs_usb | Ubuntu (WSL) | Loads the candleLight/gs_usb CAN driver and should create can0 when the adapter is attached. | fast |
| ip link show can0 | Ubuntu (WSL) | Checks whether the CAN network interface exists. | fast |
| sudo ip link set can0 up type can bitrate 1000000 | Ubuntu (WSL) | Brings up can0 at 1 Mbps, matching the tested configuration. Confirm the required bitrate for your robot/adapter. | fast |
| ip -details link show can0 | Ubuntu (WSL) | Verifies that can0 is UP and reports CAN link details. | fast |
| python3 -m venv ~/piper-env | Ubuntu (WSL) | Creates an isolated Python environment. | fast |
| source ~/piper-env/bin/activate | Ubuntu (WSL) | Activates the environment; the prompt should show (piper-env). | fast |
| python -m pip install --upgrade pip | Ubuntu (WSL), venv active | Updates pip in the environment. | Fast to 1 minute |
| python -m pip install python-can | Ubuntu (WSL), venv active | Installs Python CAN support. | Fast to 1 minute |
| python -m pip install "git+https://github.com/agilexrobotics/pyAgxArm.git" | Ubuntu (WSL), venv active | Installs pyAgxArm from the AgileX GitHub repository. | 1–3 minutes |
| python -c "import pyAgxArm; print('pyAgxArm imported successfully')" | Ubuntu (WSL), venv active | Checks that pyAgxArm imports. | fast |
| python -c "from pyAgxArm import create_agx_arm_config, AgxArmFactory, ArmModel, PiperFW; r=AgxArmFactory.create_arm(create_agx_arm_config(robot=ArmModel.PIPER, firmeware_version=PiperFW.DEFAULT, interface='socketcan', channel='can0')); r.connect(); print(r.get_arm_status()); print(r.get_joint_angles())" | Ubuntu (WSL), venv active | Connects to the PiPER and reads status and joint angles. This is a read-only test. | fast |
| python -c "from pyAgxArm import create_agx_arm_config, AgxArmFactory, ArmModel, PiperFW; import time; r=AgxArmFactory.create_arm(create_agx_arm_config(robot=ArmModel.PIPER, firmeware_version=PiperFW.DEFAULT, interface='socketcan', channel='can0')); r.connect(); exec('while True:\n print(r.get_joint_angles())\n time.sleep(0.5)')" | Ubuntu (WSL), venv active | Continuously prints joint-angle feedback every 0.5 seconds; stop with Ctrl+C. Read-only. | Run until Ctrl+C |

# Verification results from the completed setup

The custom WSL kernel loaded successfully and the gs_usb driver created can0.

The can0 interface was brought UP at 1,000,000 bit/s.

pyAgxArm imported successfully and connected using SocketCAN on can0.

The arm reported NORMAL status and returned joint angles.

Repeated joint-angle feedback was received at 0.5-second intervals without issuing motion commands.

INSTALLATION IS COMPLETE. The software installation and read-only communication verification are finished. Actual movement commands and physical motion testing have not been completed.

# Prompt for future AI assistance

Use this document as the installation record for setting up an AgileX PiPER 6-DOF robot arm from Windows using WSL2 Ubuntu, a USB-CAN adapter, SocketCAN, and pyAgxArm without ROS or ROS2. Walk me through the process one command at a time. Keep explanations concise, tell me which terminal to use, wait for me to report the result before giving the next command, and adapt if my output differs. Do not assume a USB BUSID or Windows username; help me identify the current values. Distinguish commands that are prerequisites from commands that can be skipped when already completed. Do not run or recommend any command or example that could physically move the robot or gripper without first warning me clearly and getting my confirmation. Prefer read-only checks until I explicitly approve a movement test. At the end, summarize which installation and verification steps succeeded and which remain.